EMPATHOME LEARNING FOUNDATION

HYBRID LAN DOCUMENT - Trevor Ian Flickinger

Table of Contents:

1) Internal router configuration

2) DMZ router configuration

3) VPN configuration

4) Azure Resources

5) LAN Topology

6) Wazuh Installation (SEIM)

I. INTERNAL ROUTER CONFIGURATION

// router syntax //

> enable

# configure terminal

# interface GigabitEthernet0 0

# hostname INTERNAL_RTR

# interface GigabitEthernet0 0

# description LAN_INTERFACE

# ip address 192.168.1.1 255.255.255.0

# no shutdown

# exit

# interface GigabitEthernet0 1

# description CONNECT_TO_DMZ_ROUTER

# ip address 192.168.2.1 255.255.255.252

# no shutdown

# router ospf 1

# network 192.168.1.0 0.0.0.255 area 0

# network 192.168.2.0 0.0.0.3 area 0

// end router syntax //

II. DMZ ROUTER CONFIGURATION

// router syntax //

> enable

# configure Terminal

# hostname DMZ_RTR

# interface GigabitEthernet0 0

# description OUTSIDE_INTERNET

# ip address [Public_IP] [Subnet_Mask]

# no shutdown

# interface GigabitEthernet0 1

# description DMZ_SERVER_ZONE

# ip address 192.168.10.1 255.255.255.0

# no shutdown

# interface GigabitEthernet0 2

# description CONNECT_TO_INTERNAL

# ip address 192.168.2.2 255.255.255.252

# no shutdown

// end router syntax //

III. VPN Configuration

VNET 10.0.0.0/16

GatewaySubnet

Azure object which represents the on-prem Cisco router and office subnet (192.168.1.0/24).

Connection: The resource that links the two gateways and holds the Shared Key

VPN Handshake

// router syntax //

# crypto ikev2 proposal AZURE-PROP

# encryption aes-cbc-256

# integrity sha256

# group 14

# crypto ikev2 policy AZURE-POL

# proposal AZURE-PROP

# crypto ikev2 keyring AZURE-KEY

# peer AZURE-GW

# address [Azure_Gateway_Public_IP]

# pre-shared-key Your_Strong_Secret_Key

// end router syntax //

Virtual Tunnel VTI

# interface Tunnel1

# ip address 169.254.0.1 255.255.255.252

# tunnel source [Your_Outside_Interface]

# tunnel mode ipsec ipv4

# tunnel destination [Azure_Gateway_Public_IP]

# tunnel protection ipsec profile AZURE-IPSEC-PRO

# ip tcp adjust-mss 1350

IV. Azure Resources

Resource Groups:

Storage

VMs:

Web server

Database

VNET / NSG

VI. LAN topology + DHCP config

// router syntax //

# ip dhcp excluded-address 192.168.1.1 192.168.1.20

# !

# ip dhcp pool OFFICE_LAN

# network 192.168.1.0 255.255.255.0

# default-router 192.168.1.1

# dns-server 8.8.8.8 1.1.1.1

# lease 8

# ip dhcp pool PRINTER_1

# host 192.168.1.15 255.255.255.0

# client-identifier 01aa.bbcc.ddee.ff

# ip dhcp pool PRINTER_1

# host 192.168.1.16 255.255.255.0

# client-identifier 01bb.ccdd.eeff.11

# show ip dhcp binding

// end router syntax //

Dotted Decimal	CIDR	Total Host IPs

255.255.255.0	/24	254

255.255.255.252	/30	2 (router-to-router link)

Standard issue workstations: 50

Higher end workstations: 2

machine learning workstations: 2

Network printers: 2

Total connections: 56

Maximum supported connections for the LAN (future expansion): 254

Standard config:

AMD Ryzen 5 9600X

24 GB DDR5-5600 CL28

1 TB SSD

NVIDIA RTX 5070

Dual Boot: Windows 11 Pro + Fedora

High End Config:

AMD Ryzen 7 9700X

32 GB DDR-5600 CL28

2 TB SSD

NVIDIA RTX 5070 Ti

Dual Boot: Windows 11 Pro + Fedora

Machine Learning Config:

Intel Core Ultra 9 285K

64 GB DDR-6000 CL30

4 TB SSD

NVIDIA RTX 5090

Ubuntu LTS

VII. Installing Wazuh (SEIM)

Via Ubuntu WSL:

curl -sL https://packages.wazuh.com/4.14/wazuh-install.sh -o wazuh-install.sh

wazuh-install.sh -a

ATTN: Make sure to save admin password (printed to the terminal) after installation completes so you can login to the web interface.